#CorwinRey

         Clean the entire motorcycle with a mild detergent and water solution. Remove any metal protecting agents.

         Remove any plastic or rubber covers that you put on the air box intakes, exhaust pipes and air box drain.

         Make sure the battery is in good condition and fully charged. Be sure the vent hose is connected and routed properly.

         Drain and replace the oil that was in the engine while the bike was in storage. Replace the oil filter if applicable.

         Lubricate all areas as instructed in the operator’s manual.

         Change the spark plugs.

         Check the engine coolant and be sure it is at the proper level. Replace it if necessary.

         If the bike is chain driven, inspect the chain and drive sprockets for excessive wear or loosening. Adjust if necessary.

         If your bike is shaft-driven, inspect the oil level in the final drive unit. Make sure it is filled to the proper level.

         Inspect the tires. Tires should not have cracks, bulges or excessive wear. Also tires should be inflated to the proper manufacturer’s specifications.

         Inspect the brake pads or shoes. If they show excessive wear, they should be replaced.

         If your bike is equipped with hydraulic brakes, check the fluid level to ensure it is at the proper level. Most manufacturers recommend changing this fluid at least once a season to eliminate moisture in the master brake cylinders. Moisture can cause corrosion and eventual brake failure. Follow the manufacturer’s recommendation on fluid type.

         Inspect coolant hoses and brake lines for signs of deterioration or cracking. Replace if necessary.

         Inspect all cables for ease of operation. Sticking, binding or broken cables should be replaced.

         Inspect all electrical accessories for proper operation. Check the headlight, turn signals, reverse lights, horn and kill switch. Correct any problems.

         Before starting the engine, slowly crank it over with the kick starter and listen for any abnormal noises. Also check for smooth movement.

         Do a complete inspection of the bike. Then start it and let it run for a few minutes in a well-ventilated area. If there are any abnormal noises or anything seems unusual, have your bike checked out by a factory-trained mechanic.

         Use a fuel system cleaner additive for a couple tanks.

         Take the motorcycle safety class you signed up for.

         Get ready to enjoy another riding season!